3D Printing Summary

| Metrics |  |
| --- | --- |
| Total Print Time (min) | 1:23 |
| Total Number of Components | 6 |
| Typical Total Mass (g) | 7 |
| Typical Number of Print Setups | 1 |

3D Printing Settings

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Finger Lift Lever | 1 | 0:19 | 2 | 15 | N | 0.2/0.4 | - Print in orientation given in STL |
| Balance Screw Holder | 1 | 0:07 | 1 | 15 | N | 0.2/0.4 | - Print in orientation given in STL |
| Counterweight nut holder | 1 | 0:11 | 1 | 15 | N | 0.2/0.4 | - Print in orientation given in STL |
| Bearing Base & Bearing Base Cap | 1 | 0:29 | 2 | 15 | N | 0.2/0.4 | - Print in orientation given in STL |
| Reed Switch Plate | 1 | 0:10 | 1 | 15 | N | 0.2/0.4 | - Print in orientation given in STL |

Customization Options

Specific colours are not a requirement for this device, so device (or components) may be printed in whichever colours the user desires.

Post-Processing

Remove any stranding or excess material in holes with a hobby knife. Any parts of the Finger Lift Pad that feel rough or scratchy should be lightly sanded with fine sandpaper until smooth to touch.

Examples of Quality Prints

Finger Lift Lever

Bearing Base

Reed Switch Plate

Bearing Base Cap

Balance Screw Holder

Counterweight nut holder